Excavation permit

Permit to dig and excavation checklist

No excavation may start until this permit is authorised. Plans of underground services are indicative only: locate services physically before machine excavation near them. Excavations in or near a shaft or trench deeper than 1.5 metres are high-risk construction work and require a SWMS.

1. Excavation details

| Permit number |  | Site |  |
| --- | --- | --- | --- |
| Location |  |  |  |
| Purpose Footings, stormwater, services, pool, retaining wall |  |  |  |
| Length x width x depth (planned) |  | Method Machine, hand, vacuum |  |
| Excavation contractor |  | Plant operator |  |
| Start date |  | Expected backfill date |  |
| SWMS reference (required if deeper than 1.5 m) |  | Soil / geotechnical report reference |  |

2. Underground services

| # | Check | Yes | No | N/A | Details / reference |
| --- | --- | --- | --- | --- | --- |
| 1 | Before You Dig Australia enquiry lodged and plans received for all asset owners | ☐ | ☐ | ☐ |  |
| 2 | Plans current for the dates of the work | ☐ | ☐ | ☐ |  |
| 3 | Site plans, previous works and visible services (pits, meters, markers) checked | ☐ | ☐ | ☐ |  |
| 4 | Services located on the ground with a locator | ☐ | ☐ | ☐ |  |
| 5 | Services exposed by hand or vacuum excavation before machine digging nearby | ☐ | ☐ | ☐ |  |
| 6 | Services marked on the ground and shown to the operator | ☐ | ☐ | ☐ |  |
| 7 | Asset owner requirements for work near their services noted and followed | ☐ | ☐ | ☐ |  |
| 8 | Overhead powerlines near plant identified and controlled | ☐ | ☐ | ☐ |  |

3. Services identified

| Service | Owner | Located by / method | Depth found | Controls |
| --- | --- | --- | --- | --- |
| EXAMPLE Gas service | Gas network | Locator, then potholed by hand |  | Hand dig within the owner's specified distance; operator briefed |

4. Ground support, spoil and access

| # | Check | Yes | No | N/A | Details |
| --- | --- | --- | --- | --- | --- |
| 1 | Ground support method chosen by a competent person: benching, battering or shoring | ☐ | ☐ | ☐ |  |
| 2 | Shoring or trench shield designed or certified where required | ☐ | ☐ | ☐ |  |
| 3 | Water in the excavation controlled (pumping, diversion) | ☐ | ☐ | ☐ |  |
| 4 | Spoil placed back from the edge, not surcharging the side | ☐ | ☐ | ☐ |  |
| 5 | Plant and vehicles kept back from the edge | ☐ | ☐ | ☐ |  |
| 6 | Nearby structures, footings and fences assessed for undermining | ☐ | ☐ | ☐ |  |
| 7 | Barriers or covers around the excavation, with signage | ☐ | ☐ | ☐ |  |
| 8 | Safe access and egress: ladder or ramp at suitable spacing | ☐ | ☐ | ☐ |  |
| 9 | No one in the excavation while plant is digging | ☐ | ☐ | ☐ |  |
| 10 | Atmosphere checked if the excavation could collect gas or lack oxygen | ☐ | ☐ | ☐ |  |
| 11 | Rescue arrangements planned | ☐ | ☐ | ☐ |  |

5. Authorisation

Services have been located and marked and the controls above are in place.

| Authorised by |  | Signature |  |
| --- | --- | --- | --- |
| Date and time |  | Valid until |  |
| Operator: I have been shown the services and controls (signature) |  |  |  |

6. Daily and after-event inspections

Inspect before each shift, after rain, and after any event that could affect stability.

| Date | Time | Condition (walls, water, edges, support) | Safe to enter? | Inspected by |
| --- | --- | --- | --- | --- |

7. Close-out

| Backfilled / made safe (date) |  | Services reinstated and as-built recorded? |  |
| --- | --- | --- | --- |
| Permit closed by |  | Date |  |