Practical completion and handover checklist

Pre-handover inspection for residential building work

Complete the builder's own inspection first, then a joint inspection with the owner. Record defects in section 7 with agreed rectification dates. A new building must not be occupied until the occupation certificate has been issued.

1. Project details

| Site address |  | Owner |  |
| --- | --- | --- | --- |
| Contract / job number |  | Principal certifier |  |
| Builder's inspection date |  | Joint inspection date |  |
| Practical completion date (proposed) |  | Inspected by |  |

2. Exterior

| # | Check | Yes | No | N/A | Comments / action |
| --- | --- | --- | --- | --- | --- |
| 1 | Roof covering complete, no damaged or loose sheets or tiles | ☐ | ☐ | ☐ |  |
| 2 | Gutters, downpipes and stormwater connected and falling correctly | ☐ | ☐ | ☐ |  |
| 3 | Flashings and cappings complete | ☐ | ☐ | ☐ |  |
| 4 | Cladding and brickwork complete, clean, no damage | ☐ | ☐ | ☐ |  |
| 5 | External sealants and joints complete | ☐ | ☐ | ☐ |  |
| 6 | Windows and external doors operate, seal and lock | ☐ | ☐ | ☐ |  |
| 7 | Weep holes and vents clear | ☐ | ☐ | ☐ |  |
| 8 | Paths, driveway and paving complete, falls away from the house | ☐ | ☐ | ☐ |  |
| 9 | Site drainage and surface water directed away from the building | ☐ | ☐ | ☐ |  |
| 10 | Fencing, gates and pool fencing complete (pool fencing certified if applicable) | ☐ | ☐ | ☐ |  |
| 11 | Letterbox and house number installed | ☐ | ☐ | ☐ |  |
| 12 | Landscaping and site clean-up complete | ☐ | ☐ | ☐ |  |

3. Interior (every room)

| # | Check | Yes | No | N/A | Comments / action |
| --- | --- | --- | --- | --- | --- |
| 1 | Ceilings and cornices: no cracks, joints invisible, paint complete | ☐ | ☐ | ☐ |  |
| 2 | Walls: finish even in raking light, no damage, paint complete | ☐ | ☐ | ☐ |  |
| 3 | Floors and coverings: complete, level, no damage, trims fitted | ☐ | ☐ | ☐ |  |
| 4 | Doors: hung true, even gaps, latch and close properly, stops fitted | ☐ | ☐ | ☐ |  |
| 5 | Windows: operate, lock, clean, flyscreens fitted | ☐ | ☐ | ☐ |  |
| 6 | Joinery and cabinetry: aligned, drawers and doors adjusted, benchtops undamaged | ☐ | ☐ | ☐ |  |
| 7 | Tiling: grout complete and even, no drummy tiles, silicone to junctions | ☐ | ☐ | ☐ |  |
| 8 | Wet areas: falls to waste, no ponding, screens sealed | ☐ | ☐ | ☐ |  |
| 9 | Fixtures and fittings: installed, secure, match selections | ☐ | ☐ | ☐ |  |
| 10 | Wardrobes and shelving complete | ☐ | ☐ | ☐ |  |
| 11 | Stairs, balustrades and handrails complete and secure | ☐ | ☐ | ☐ |  |
| 12 | All protective films and labels removed; builder's clean complete | ☐ | ☐ | ☐ |  |

4. Services testing

| # | Check | Yes | No | N/A | Comments / action |
| --- | --- | --- | --- | --- | --- |
| 1 | Every power point and light switch tested | ☐ | ☐ | ☐ |  |
| 2 | Smoke alarms installed, interconnected where required, tested | ☐ | ☐ | ☐ |  |
| 3 | Safety switches (RCDs) tested | ☐ | ☐ | ☐ |  |
| 4 | Hot water system working and set correctly | ☐ | ☐ | ☐ |  |
| 5 | All taps, showers and toilets working, no leaks | ☐ | ☐ | ☐ |  |
| 6 | Drainage flows freely, no smells | ☐ | ☐ | ☐ |  |
| 7 | Exhaust fans working and ducted to outside | ☐ | ☐ | ☐ |  |
| 8 | Air conditioning and heating commissioned | ☐ | ☐ | ☐ |  |
| 9 | Appliances installed and working | ☐ | ☐ | ☐ |  |
| 10 | Gas appliances commissioned | ☐ | ☐ | ☐ |  |
| 11 | Solar, battery and EV charger commissioned (if installed) | ☐ | ☐ | ☐ |  |
| 12 | Data, TV and intercom working | ☐ | ☐ | ☐ |  |
| 13 | Garage door and remotes working | ☐ | ☐ | ☐ |  |
| 14 | Water, electricity and gas meters read and recorded | ☐ | ☐ | ☐ |  |

5. Certificates and approvals

| # | Check | Received | N/A | Reference / date |
| --- | --- | --- | --- | --- |
| 1 | Occupation certificate from the principal certifier | ☐ | ☐ |  |
| 2 | Electrical compliance certificate | ☐ | ☐ |  |
| 3 | Plumbing and drainage compliance certificate | ☐ | ☐ |  |
| 4 | Gas compliance certificate | ☐ | ☐ |  |
| 5 | Waterproofing certificate(s) | ☐ | ☐ |  |
| 6 | Termite management certificate and durable notice | ☐ | ☐ |  |
| 7 | Glazing certificate | ☐ | ☐ |  |
| 8 | Smoke alarm compliance | ☐ | ☐ |  |
| 9 | BASIX or energy commitments evidence | ☐ | ☐ |  |
| 10 | Pool fence or pool compliance (if applicable) | ☐ | ☐ |  |
| 11 | Structural engineer certification (if required) | ☐ | ☐ |  |
| 12 | Home warranty insurance certificate (where required) | ☐ | ☐ |  |

6. Handover pack

| # | Check | Given | Details |
| --- | --- | --- | --- |
| 1 | All keys, including window keys, counted | ☐ |  |
| 2 | Remotes and access fobs | ☐ |  |
| 3 | Appliance manuals and warranties | ☐ |  |
| 4 | Product warranties (roofing, waterproofing, hot water, tapware) | ☐ |  |
| 5 | Copies of certificates and the occupation certificate | ☐ |  |
| 6 | Maintenance guide for the home | ☐ |  |
| 7 | Paint, tile and product selection schedule | ☐ |  |
| 8 | Emergency and warranty contact details | ☐ |  |
| 9 | Statutory warranty and defects information | ☐ |  |

7. Items raised at the inspection

| # | Location | Item | Agreed action | Due | Done |
| --- | --- | --- | --- | --- | --- |
| EXAMPLE 1 | Ensuite | Shower screen door rubs on seal when closing | Adjust hinges, replace seal if damaged | 7 days |  |

8. Acknowledgement

The parties have inspected the work together. Items listed in section 7 will be completed by the dates shown.

| Builder |  | Signature and date |  |
| --- | --- | --- | --- |
| Owner |  | Signature and date |  |