Scaffold inspection checklist

Handover, periodic and after-event inspections

To be completed by a competent person. Do not use a scaffold that is incomplete, altered without a licensed scaffolder, or tagged unsafe. Alterations must be made only by a person licensed for the type of scaffold.

1. Scaffold details

| Site |  | Scaffold location / ID |  |
| --- | --- | --- | --- |
| Type Modular, tube and coupler, mobile, hung, cantilever |  | Duty rating Light, medium or heavy duty |  |
| Erected by (company) |  | Scaffolder licence number |  |
| Handover certificate received (date) |  | Design / engineer certificate (if required) |  |

2. Reason for inspection

| ☐ Handover before first use | ☐ Routine interval | ☐ After an alteration |
| --- | --- | --- |
| ☐ After an incident | ☐ After repairs | ☐ After severe weather |
| ☐ Other: ____________________ |  |  |

3. Foundations and base

| # | Check | Yes | No | N/A | Comments / action |
| --- | --- | --- | --- | --- | --- |
| 1 | Ground firm, level and drained | ☐ | ☐ | ☐ |  |
| 2 | Sole boards and base plates under every standard | ☐ | ☐ | ☐ |  |
| 3 | Adjustable bases within their extension limits | ☐ | ☐ | ☐ |  |
| 4 | No excavation or undermining near standards | ☐ | ☐ | ☐ |  |
| 5 | Mobile scaffold castors locked and on level ground | ☐ | ☐ | ☐ |  |

4. Structure

| # | Check | Yes | No | N/A | Comments / action |
| --- | --- | --- | --- | --- | --- |
| 1 | Standards plumb and correctly spaced | ☐ | ☐ | ☐ |  |
| 2 | Ledgers and transoms level and in place | ☐ | ☐ | ☐ |  |
| 3 | Bracing complete as designed (longitudinal and transverse) | ☐ | ☐ | ☐ |  |
| 4 | Couplers and connections secure | ☐ | ☐ | ☐ |  |
| 5 | Ties or anchors installed at the designed spacing and secure | ☐ | ☐ | ☐ |  |
| 6 | No components damaged, bent or corroded | ☐ | ☐ | ☐ |  |
| 7 | No unauthorised alterations or missing components | ☐ | ☐ | ☐ |  |

5. Working platforms and edge protection

| # | Check | Yes | No | N/A | Comments / action |
| --- | --- | --- | --- | --- | --- |
| 1 | Platforms full width with no gaps, secured against movement | ☐ | ☐ | ☐ |  |
| 2 | Planks or decks in good condition, correctly supported and lapped | ☐ | ☐ | ☐ |  |
| 3 | Guardrails on all open sides and ends | ☐ | ☐ | ☐ |  |
| 4 | Mid-rails fitted | ☐ | ☐ | ☐ |  |
| 5 | Toe boards or mesh fitted | ☐ | ☐ | ☐ |  |
| 6 | Gap between platform and building face protected | ☐ | ☐ | ☐ |  |
| 7 | Platform not overloaded with materials for its duty rating | ☐ | ☐ | ☐ |  |
| 8 | Containment mesh or screens intact where fitted | ☐ | ☐ | ☐ |  |

6. Access and surroundings

| # | Check | Yes | No | N/A | Comments / action |
| --- | --- | --- | --- | --- | --- |
| 1 | Stair or ladder access secured and in good condition | ☐ | ☐ | ☐ |  |
| 2 | Access gates or hatches closed | ☐ | ☐ | ☐ |  |
| 3 | Clearance from overhead powerlines maintained | ☐ | ☐ | ☐ |  |
| 4 | Area below protected: exclusion zone, hoarding or catch fans | ☐ | ☐ | ☐ |  |
| 5 | Signage and scaffold tag displayed at every access point | ☐ | ☐ | ☐ |  |
| 6 | Adjacent loads (materials, plant) not affecting the scaffold | ☐ | ☐ | ☐ |  |

7. Result

| ☐ Safe to use: tag updated | ☐ Restricted use (details on tag) | ☐ Unsafe: tagged out, do not use |
| --- | --- | --- |

8. Defects found and action

| Defect | Location | Action / who | Fixed (date) |
| --- | --- | --- | --- |

9. Inspector

| Name |  | Competency / licence |  |
| --- | --- | --- | --- |
| Signature |  | Date and time |  |
| Next inspection due |  | Tag updated? |  |

10. Inspection register

| Date | Scaffold ID | Reason | Result | Inspector | Next due |
| --- | --- | --- | --- | --- | --- |